Project Background

The project is located in Guyuan City in northwest China, an arid region with an average annual precipitation below 450 mm. However, climate change has intensified rainfall events, exceeding the capacity of the existing urban drainage system and causing frequent flooding. During heavy rainstorms, surface runoff from nearly half of the urban area converges into a single low-lying area, resulting in ponding along a 1-kilometer road corridor with water depths exceeding 30 cm. The collapsible loess soil further increases the risk of road subsidence and structural damage.

Flooding Analysis

Traditionally, urban flooding is addressed by upgrading drainage systems to accommodate 5- to 50-year storms; however, this approach was dismissed due to high cost and long construction periods. High-resolution LiDAR-based modeling revealed that flooding was primarily caused by disrupted overland flow paths between the depressed urban area and the Qingshui River, resulting from the elevated riverbank.

Green Infrastructure Solution

In collaboration with drainage engineers, a resilient artificial stream was implemented as a green infrastructure solution and integrated into a riverside park. The landscape was designed to manage 50-year storm events while supporting ecological functions and public use. The project was realized through an integrated process of flood modeling, landscape design, and construction, supported by finely detailed cross-sectional designs balancing drainage capacity, ecological performance, structural stabilization, and multi-use public space.